| CHEMTECH PROJECT NUMBER: P4103 | MATRIX: TCLP |
| --- | --- |
| METHOD: 8151A/3510/1311 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for 365-351-294MS [2,4-DCAA(1) - 55%, 2,4-DCAA(2) - 62%], 365-351-294MSD [2,4-DCAA(1) - 56%, 2,4-DCAA(2) - 57%], WC-22A [2,4-DCAA(1) - 66%, 2,4-DCAA(2) - 67%], WC-14-5-7.5A [2,4-DCAA(1) - 63%, 2,4-DCAA(2) - 60%], WC-14-5-7.5B [2,4-DCAA(1) - 66%, 2,4-DCAA(2) - 68%], WC-14-7.5-10A [2,4-DCAA(1) - 63%, 2,4-DCAA(2) - 57%], WC-14-7.5-10B [2,4-DCAA(1) - 67%, 2,4-DCAA(2) - 68%], WC-23 [2,4-DCAA(1) - 63%, 2 and4-DCAA(2) - 59%] these compound did not meet the NJDKQP criteria but met the in-house criteria therefor no corrective action taken. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS recoveries met the requirements for all compounds . The MSD recoveries met the acceptable requirements . The Blank Spike met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

_______________________________________				__________________________

QA REVIEW								Date