| CHEMTECH PROJECT NUMBER: P4549 | MATRIX: Water |
| --- | --- |
| METHOD: 8082A/3510 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. The Initial Calibration met the requirements . The Continuous Calibration met the requirements . |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for TT-067-IDWGW-20241024 [Decachlorobiphenyl(1) - 34%, Decachlorobiphenyl(2) - 32%], TT-067-IDWGW-20241024RE [Decachlorobiphenyl(1) - 39%, Decachlorobiphenyl(2) - 37%], TT-068-IDWGW-20241024 [Decachlorobiphenyl(1) - 29%, Decachlorobiphenyl(2) - 28%], TT-068-IDWGW-20241024RE [Decachlorobiphenyl(1) - 33%, Decachlorobiphenyl(2) - 32%], TT-069-IDWGW-20241024 [Decachlorobiphenyl(1) - 30%, Decachlorobiphenyl(2) - 28%], TT-069-IDWGW-20241024RE [Decachlorobiphenyl(1) - 34% and Decachlorobiphenyl(2) - 32%], All the failure samples in surrogates with both columns were reanalyzed to confirm the results as per method and reported in the data. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Blank Spike met requirements for all samples . The Blank Spike Duplicate met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

The laboratory certifies that the all-electronic diskette deliverable exactly match the data summary forms (i.e. Form Is).”

The not QT review data is reported in the Miscellaneous.

_______________________________________				__________________________

QA REVIEW								Date