| CHEMTECH PROJECT NUMBER: P4602 | MATRIX: Solid |
| --- | --- |
| METHOD: 8151A/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. The Initial Calibration met the requirements . The Continuous Calibration File ID PS028237.D met the requirements except for 2,4,5-T,2,4,5-TP (Silvex),2,4-D,2,4-DB,2,4-DCAA,DICAMBA,DICHLORPROP,DINOSEB is failing in 1st column but it is passing in 2nd column also associated QC passing for that therefore no corrective action taken |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS {P4602-44MS} with File ID: PS028408.D recoveries met the requirements for all compounds except for Dinoseb[0%],due to matrix interference. The MS {P4602-41MS} with File ID: PS028411.D recoveries met the requirements for all compounds except for Dinoseb[0%],due to matrix interference. The MS {P4602-38MS} with File ID: PS028436.D recoveries met the requirements for all compounds except for 2,4,5-TP(Silvex)[7%] and Dinoseb[8%],due to matrix interference. The MSD {P4602-45MSD} with File ID: PS028409.D recoveries met the acceptable requirements except for Dinoseb[0%],due to matrix interference. The MSD {P4602-42MSD} with File ID: PS028412.D recoveries met the acceptable requirements except for Dinoseb[0%],due to matrix interference. The MSD {P4602-39MSD} with File ID: PS028437.D recoveries met the acceptable requirements except for 2,4,5-TP(Silvex)[6%] and Dinoseb[8%],due to matrix interference. The Blank Spike met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. The Holding Times were met for all samples except for C0P17, CC0P2, CC0P4, CC0P6, CC0P8, CC0Q0, CC0Q2, CC0Q3, CC0Q4, CC0Q5, CC0Q7, CC0Q9, CC0R0, CC0R1, CC0R2 and CC0P0. |  |  |  |

ADDITIONAL COMMENTS:

The soil samples  results  are based on a dry weight basis.

_______________________________________				__________________________

QA REVIEW								Date