| CHEMTECH PROJECT NUMBER: P4663 | MATRIX: Solid |
| --- | --- |
| METHOD: 8151A/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for CC0R8MS [2,4-DCAA(2) - 51%], CC0R8MSD [2,4-DCAA(2) - 51%], TENNANT-PAD-2-3-STOCKPILE [2,4-DCAA(1) - 56%, 2,4-DCAA(2) - 51%], RES-BUILDING-PAD-NO-11 [2,4-DCAA(1) - 54%, 2,4-DCAA(2) - 49%], RES-BUILDING-PAD-NO-3 [2,4-DCAA(1) - 50%, 2,4-DCAA(2) - 39%], RES-BUILDING-PAD-NO-2 [2 and4-DCAA(2) - 66%]these compounds did not meet the NJDKQP criteria but met the in-house criteria . |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS {P4602-44MS} with File ID: PS028408.D recoveries met the requirements for all compounds except for 2,4,5-T[27%], 2,4,5-TP(Silvex)[16%], 2,4-D[50%], 2,4-DB[29%], DICAMBA[43%], DICHLORPROP[26%]these compounds did not meet the NJDKQP criteria but met the in-house criteria and Dinoseb[0%]this compound did not meet the NJDKQP criteria and in-house criteria due to sample matrix interference. The MSD {P4602-45MSD} with File ID: PS028409.D recoveries met the acceptable requirements except for 2,4,5-T[28%], 2,4,5-TP(Silvex)[16%], 2,4-D[50%], 2,4-DB[29%], DICAMBA[43%], DICHLORPROP[26%]these compounds did not meet the NJDKQP criteria but met the in-house criteria and Dinoseb[0%]this compound did not meet the NJDKQP criteria and in-house criteria due to sample matrix interference. The Blank Spike met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

Fax and Hardcopy data will not match for all samples as Fax samples analyzed in sequence PS110424 where CCAL having F flag as a corrective action sample reanalyzed and reported in hardcopy and fax data From sequence PS110424 provided in Miscellaneous Section.

The soil samples  results  are based on a dry weight basis.

_______________________________________				__________________________

QA REVIEW								Date