| CHEMTECH PROJECT NUMBER: P4721 | MATRIX: Solid |
| --- | --- |
| METHOD: NJEPH/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for EX-2-TPH-1DL [2-Bromonaphthalene (SURR) - 180%], EX-2-TPH-2DL [2-Bromonaphthalene (SURR) - 155%], EX-2-TPH-3 [2-Bromonaphthalene (SURR) - 140%], EX-2-TPH-3DL [2-Bromonaphthalene (SURR) - 160%], EX-11-TPH-1 [2-Bromonaphthalene (SURR) - 149%], EX-11-TPH-1DL [2-Bromonaphthalene (SURR) - 170%, 2-Flurobiphenyl (SURR) - 146%], EX-11-TPH-1DL2 [2-Bromonaphthalene (SURR) - 174%], EX-11-TPH-2 [2-Bromonaphthalene (SURR) - 150%], EX-11-TPH-2DL [2-Bromonaphthalene (SURR) - 181%], EX-11-TPH-4 [2-Bromonaphthalene (SURR) - 163%, 2-Flurobiphenyl (SURR) - 147%], EX-11-TPH-4DL [2-Bromonaphthalene (SURR) - 183%, 2-Flurobiphenyl (SURR) - 165%], EX-11-TPH-4DL2 [2-Bromonaphthalene (SURR) - 208%, 2-Flurobiphenyl (SURR) - 188%], EX-9-TPH-7DL2 [2-Bromonaphthalene (SURR) - 168% and 2-Flurobiphenyl (SURR) - 165%]due to high concentration sample required dilution therefore samples were reanalyzed with dilution and reported. while,EX-2-TPH-1DL2 [1-chlorooctadecane (SURR) - 0%],EX-11-TPH-2DL [1-chlorooctadecane (SURR) - 0%],EX-11-TPH-4DL2 [1-chlorooctadecane (SURR) - 0%],Surrogates were diluted out due to the high dilution No further corrective action was taken. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS recoveries met the requirements for all compounds . The MSD recoveries met the acceptable requirements . The Blank Spike met requirements for all samples . The Blank Spike Duplicate met requirements for all samples . The RPD for {P4718-01MSD} with File ID: FD048679.D met criteria except for Aromatic C12-C16[28.9%]due to difference in results of MS-MSD. |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

Samples EX-11-TPH-2 was diluted due to bad matrix.

Samples EX-2-TPH-1, EX-2-TPH-2, EX-2-TPH-3, EX-11-TPH-1, EX-11-TPH-2, EX-11-TPH-3, EX-11-TPH-4, EX-9-TPH-4 and EX-9-TPH-7 EX-2-TPH-1DL, EX-2-TPH-2DL, EX-11-TPH-1DL, EX-11-TPH-4DL, EX-9-TPH-7DL were diluted due to high concentrations.

The soil samples  results  are based on a dry weight basis.

_______________________________________				__________________________

QA REVIEW								Date