| CHEMTECH PROJECT NUMBER: P4722 | MATRIX: Solid |
| --- | --- |
| METHOD: 1030,160.4,6010D,7196A,9012B,9034,9045D,9071B,9095B,SM2540 B,SM4500-NH3,SM5220 D |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 2. | Matrix Spike Duplicate Recoveries Met Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Blank Spike met requirements for all samples.The Matrix Spike (WC-3(0-6)MS) analysis met criteria for all samples except for Oil and Grease due to matrix interferences. The Matrix Spike Duplicate (WC-3(0-6)MSD) analysis met criteria for all samples except for Oil and Grease due to matrix interferences. |  |  |  |
| 3. | Sample Duplicate Analysis Met QC Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Duplicate (WC-3(0-6)DUP) analysis met criteria for all samples except for Ammonia as N & The Duplicate (WC-3(0-6)DUP) analysis met criteria for all samples except for Reactive Cyanide Due to the results are below RL. |  |  |  |
| 8. | Digestion Holding Time Met If not met, list number of days exceeded for each sample: The Holding Times were met for all samples except for WC-1(0-6) of Corrosivity & for WC-2(0-6) of Corrosivity & for WC-3(0-6) of Corrosivity due to sample receive out of hold. |  |  |  |

ADDITIONAL COMMENTS: For COD, sample P4722-03 reported with straight 5X dilution due to the original sample was reading over range, only 5X has been reported.

_______________________________________				__________________________

QA REVIEW								Date