SDG NARRATIVE

### USEPA

### SDG # MC0D37

### CASE # 51863

CONTRACT # 68HERH20D0011

SOW# SFAM01.1

### LAB NAME: Alliance Technical Group, LLC

### LAB CODE: ACE

### LAB ORDER ID # P4755

A. Number of Samples and Date of Receipt

15 Soil samples was delivered to the laboratory intact on 11/07/2024, 11/08/2024

B. Parameters

# Test requested for TCLP Metals Group1 = Cadmium, Lead.

# C. Cooler Temp

Indicator Bottle: Presence/Absence

Cooler:  2.6oC, 2.8oC, 2.4oC, 1.9oC

# D. Detail Documentation (related to Sample Handling

Shipping, Analytical Problem, Temp of Cooler etc):

Issue 1 : A "P" or "M" prefix was listed at the beginning of a CLP sample ID.

Issue 2: Laboratory QC has not been performed for SDG MC0D37 (TCLP ICP‐AES 1‐4 Metals with a 14‐day TAT) and SDG MC0DA8 (ICP‐AES 1‐4 Metals with a 7‐day TAT). The samples designated on the attached COC for Laboratory QC were already used for other SDGs. The laboratory would like to use samples MC0DA9 and MC0DA8 for QC analysis and has confirmed that these samples are not blanks, rinsates or PE samples.

E. Corrective Action taken for above:

Resolution 1 : To maintain COC integrity, ASB requests no changes to the Sample IDs. The laboratory will note the issue in the SDG Narrative and proceed with the analysis of the samples.

Resolution 2: Per Region 3, the laboratory should use samples MC0DA9 and MC0DA8 for QC analysis, note the issue in the SDG Narrative and proceed with the analysis of the samples.

F. Analytical Techniques:

All analyses were based on CLP Methodology by method SFAM01.1.

Inter Element correction factors (IECs) are determined annually and correction factor are applied during

ICP-AES analysis.

G. Calculation:

Calculation for ICP-AES Water Sample:

Concentration or Result (μg/L) =     C  x    Vf    x  DF x 1000

Vi

Where,

C  = Instrument value in ppm (The average of all replicate exposures)

Vf = Final digestion volume (mL)

Vi = Initial aliquot amount (mL) (Sample amount taken in prep)

DF = Dilution Factor

Example Calculation For Sample MC0D37 For Cadmium:

If C    = 0.0010947 ppm

Vf = 50 ml

Vi  = 50 ml

DF = 1

Concentration or Result (μg/L) =   0.0010947 x   50   x 1 x 1000

50

=  1.0947 μg/L

=  1.1 μg/L  (Reported Result with Signification)

H. QA/ QC

Calibrations met requirements.  Interference check met requirements.  Blank analyses did not indicate any

presence of contamination. Laboratory Control sample was within control limits. Spike sample did meet requirements. Duplicate sample did meet. Serial Dilution did meet requirements.

I certify that the data package is in compliance with the terms and conditions of the contract both technically and for completeness, for other than the conditions detailed above.  Release of the data contained in this hard copy data package has been authorized by the Laboratory Director or his designee, as verified by the following signature.

Signature__________________________	Name: Nimisha Pandya

Date _____________________________ 	Title: Document Control Officer