| CHEMTECH PROJECT NUMBER: P5225 | MATRIX: Solid |
| --- | --- |
| METHOD: 8082A/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for 2A-2B-2C-MB [Decachlorobiphenyl(2) - 181%],AS per method one surrogate allowed to fail to meet the criteria per column. No further corrective action was taken while, 3A-3B-3C-ABDL [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0%, Tetrachloro-m-xylene(2) - 0%],Surrogates were diluted out due to the high dilution. No further corrective action was taken while, 4A-4B-4C-BASIC [Decachlorobiphenyl(1) - 346%, Decachlorobiphenyl(2) - 393%], 4A-4B-4C-BASICDL [Decachlorobiphenyl(1) - 370% and Decachlorobiphenyl(2) - 391%]Due to high concentration sample required dilution. Therefore, sample was reanalyzed with dilution and reported. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Blank Spike met requirements for all samples . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

Samples 3A-3B-3C-AB, 4A-4B-4C-BASIC were diluted due to high concentrations.

The temperature of the samples at the time of receipt was 13.5°C.

All samples received as Caulk matrix, therefore less volume was taken at the time of extraction.

No MSMSD performed as samples are caulk matrix.

_______________________________________				__________________________

QA REVIEW								Date