| CHEMTECH PROJECT NUMBER: Q1316 | MATRIX: Water |
| --- | --- |
| METHOD: 1664A,SM2540 D,SM4500-NH3,SM5210 B |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 2. | Matrix Spike Duplicate Recoveries Met Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Blank Spike met requirements for all samples.The Matrix Spike analysis met criteria for all samples. The Matrix Spike Duplicate (DSN002MSD) analysis met criteria for all samples except for Ammonia due to sample matrix interference. |  |  |  |
| 3. | Sample Duplicate Analysis Met QC Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |
| 8. | Digestion Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |

ADDITIONAL COMMENTS: As per method 1664A, MS/MSD is required to be performed with the sample analysis. However, Lab did not receive sufficient volume to perform the MS/MSD forQ1316 therefore Lab reported MS-MSD from Q1322.

_______________________________________				__________________________

QA REVIEW								Date