| CHEMTECH PROJECT NUMBER: Q1421 | MATRIX: Solid |
| --- | --- |
| METHOD: 8081B/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. The Initial Calibration met the requirements . The Continuous Calibration met the requirements . |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for P001-CLAY-CF01-01 [Tetrachloro-m-xylene(2) - 127%], P001-CLAY-CF01-01MS [Tetrachloro-m-xylene(1) - 132%, Tetrachloro-m-xylene(2) - 127%], P001-CLAY-CF01-01MSD [Tetrachloro-m-xylene(1) - 129%, Tetrachloro-m-xylene(2) - 129%],Surrogate failure confirmed with the Original sample, P001-CLAY-CF01-02 [Tetrachloro-m-xylene(1) - 136%, Tetrachloro-m-xylene(2) - 135%], P001-CLAY-CF01-02RE [Tetrachloro-m-xylene(1) - 132% and Tetrachloro-m-xylene(2) - 134%] the failure sample in surrogates with both columns was reanalyzed to confirm the results as per method and reported in the data. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS recoveries met the requirements for all compounds . The MSD recoveries met the acceptable requirements . The Blank Spike met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

_______________________________________				__________________________

QA REVIEW								Date