| CHEMTECH PROJECT NUMBER: Q1569 | MATRIX: Solid |
| --- | --- |
| METHOD: 8082A/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. The Initial Calibration met the requirements . The Continuous Calibration met the requirements . |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for C0O18MS [Decachlorobiphenyl(2) - 170%], C0O18MSD [Decachlorobiphenyl(2) - 166%] Surrogate failing for MS-MSD but it is passing for Original sample therefore no corrective action taken. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS recoveries for {Q1572-06MS} with File ID: PP070564.D met requirements for all samples except for AR1016[7313%] and AR1260[169%]due to sample matrix interference. The MSD {Q1572-06MSD} with File ID: PP070565.D recoveries met requirements for all samples except for AR1016[7777%] and AR1260[215%]due to sample matrix interference. The Blank Spike met requirements for all samples . The RPD for {Q1572-06MSD} with File ID: PP070565.D met criteria except for AR1260[24%] due to difference in results of MS-MSD. |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

The not QT review data is reported in the Miscellaneous.

The soil samples  results  are based on a dry weight basis.

_______________________________________				__________________________

QA REVIEW								Date