| CHEMTECH PROJECT NUMBER: Q1860 | MATRIX: Solid |
| --- | --- |
| METHOD: 8082A/3541 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Chromatograms Labeled/Compounds Identified. |  |  | ü |
| 2. | Standard Summary Submitted. |  |  | ü |
| 3. | Calibration - Initial Calibration performed within 30 days before sample analysis and continuing calibration performed within 24 hours of sample analysis, 12 HOURS IF 8000 SERIES METHOD. The Initial Calibration met the requirements . The Continuous Calibration File ID PO110750.D met the requirements except for Aroclor-1016(Peak-02),Tetrachloro-m-xylene is failing in 1st column but passing but passing in 2nd column therefore no corrective action taken. The Continuous Calibration File ID PO110783.D met the requirements except for Tetrachloro-m-xylene is failing in 1st column but passing but passing in 2nd column therefore no corrective action taken. |  |  |  |
| 4. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 5. | Surrogate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable ranges. The Surrogate recoveries met the acceptable criteria except for P001-SS007-01 [Decachlorobiphenyl(2) - 215%], P001-SS007-01DL [Decachlorobiphenyl(1) - 178%], P001-SS013-01DL [Decachlorobiphenyl(2) - 194%], P001-SS021-01DL [Decachlorobiphenyl(2) - 192%] as per method one surrogate is allowed to failed, therefore no corrective action was taken and P001-SS009-01 [Decachlorobiphenyl(1) - 248%, Decachlorobiphenyl(2) - 227%], P001-SS012-01 [Decachlorobiphenyl(1) - 219%, Decachlorobiphenyl(2) - 202%], P001-SS012-01DL [Decachlorobiphenyl(1) - 219%, Decachlorobiphenyl(2) - 238%], P001-SS021-01 [Decachlorobiphenyl(1) - 187%, Decachlorobiphenyl(2) - 193%], P001-SS025-01 [Decachlorobiphenyl(1) - 250%, Decachlorobiphenyl(2) - 222%], P001-SS026-01 [Decachlorobiphenyl(1) - 373%, Decachlorobiphenyl(2) - 332%], P001-SS028-01 [Decachlorobiphenyl(1) - 423%, Decachlorobiphenyl(2) - 358%], P001-SS035-01 [Decachlorobiphenyl(1) - 179%, Decachlorobiphenyl(2) - 181%], P001-SS025-01DL [Decachlorobiphenyl(1) - 225%, Decachlorobiphenyl(2) - 240%], P001-SS026-01DL [Decachlorobiphenyl(1) - 380%, Decachlorobiphenyl(2) - 418%], P001-SS033-01DL [Decachlorobiphenyl(1) - 181%, Decachlorobiphenyl(2) - 180%], P001-SS034-01DL [Decachlorobiphenyl(1) - 188%, Decachlorobiphenyl(2) - 254%], P001-SS035-01DL [Decachlorobiphenyl(1) - 196%, Decachlorobiphenyl(2) - 229%], P001-SS031-01DL [Decachlorobiphenyl(1) - 247%, Decachlorobiphenyl(2) - 283%], P001-SS028-01DL [Decachlorobiphenyl(1) - 415%, Decachlorobiphenyl(2) - 434%], P001-SS008-01 [Decachlorobiphenyl(1) - 204%, Decachlorobiphenyl(2) - 407%, Tetrachloro-m-xylene(1) - 184%], P001-SS029-01 [Tetrachloro-m-xylene(1) - 149%, Tetrachloro-m-xylene(2) - 151%], but these samples were required further dilution as well due to high concentration, therefore original and Dilution analysis were reported and no further corrective action taken also P001-SS009-01DL [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0%, Tetrachloro-m-xylene(2) - 0%], P001-SS032-01DL2 [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0%, Tetrachloro-m-xylene(2) - 0%], P001-SS035-01DL2 [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0% and Tetrachloro-m-xylene(2) - 0%], P001-SS031-01DL2 [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0%, Tetrachloro-m-xylene(2) - 0%], P001-SS008-01DL [Decachlorobiphenyl(1) - 0%, Decachlorobiphenyl(2) - 0%, Tetrachloro-m-xylene(1) - 0%, Tetrachloro-m-xylene(2) - 0%], Surrogates were diluted out due to the high dilution. No further corrective action was taken. |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The MS {Q1860-20MS} with File ID: PO110728.D recoveries met the requirements for all compounds except for AR1016[649%], AR1260[316%] due to matrix interference. The MSD {Q1860-21MSD} with File ID: PO110729.D recoveries met the acceptable requirements except for AR1016[698%], AR1260[321%] due to matrix interference. The Blank Spike met requirements for all samples . The RPD met criteria . |  |  |  |
| 7. | Retention Time Shift Meet Criteria (if applicable) Comments: |  |  |  |
| 8. | Extraction Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS:

Samples P001-SS006-01, P001-SS007-01, P001-SS008-01, P001-SS009-01, P001-SS012-01, P001-SS013-01, P001-SS016-01, P001-SS017-01, P001-SS017-02, P001-SS019-01, P001-SS020-01, P001-SS021-01, P001-SS023-01, P001-SS025-01, P001-SS026-01, P001-SS028-01, P001-SS029-01, P001-SS030-01, P001-SS031-01, P001-SS031-01DL, P001-SS032-01, P001-SS032-01DL, P001-SS033-01, P001-SS034-01, P001-SS035-01 and P001-SS035-01DL were diluted due to high concentrations.

The soil samples  results  are based on a dry weight basis.

_______________________________________				__________________________

QA REVIEW								Date