| ORDER ID: Q2301 | MATRIX: Solid |
| --- | --- |
| METHOD: 1030,160.4,1664A,9012B,9034,9045D,9071B,9095B,SM2540 B,SM4500-NH3,SM5220 D |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 2. | Matrix Spike Duplicate Recoveries Met Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Blank Spike met requirements for all samples.The Matrix Spike (WC-URBAN-FILL-CMS) analysis met criteria for all samples except for Oil and Grease due to matrix interference. The Matrix Spike Duplicate (WC-URBAN-FILL-CMSD) analysis met criteria for all samples except for Oil and Grease due to matrix interference. |  |  |  |
| 3. | Sample Duplicate Analysis Met QC Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |
| 8. | Digestion Holding Time Met If not met, list number of days exceeded for each sample: The Holding Times were met for all samples except for WC-URBAN-FILL-C of pH, for WC-URBAN-FILL-C of Corrosivity as these samples are received out of holding time. |  |  |  |

ADDITIONAL COMMENTS: As per method 1664A, MS/MSD is required to be performed with the sample analysis. However, Lab did not receive sufficient volume to perform

the MS/MSD therefore MS/MSD were not performed for this project.

_______________________________________				__________________________

QA REVIEW								Date