| ORDER ID: Q2481 | MATRIX: TCLP |
| --- | --- |
| METHOD: 6010D,7470A,1311 |  |

|  |  | NA | NO | YES |
| --- | --- | --- | --- | --- |
| 1. | Calibration Summary met criteria. |  |  |  |
| 2. | ICP Interference Check Sample Results Summary Submitted. |  |  |  |
| 3. | Serial Dilution Summary (if applicable) Submitted. |  |  |  |
| 4. | Laboratory Control Sample Summary (if applicable) Submitted. |  |  |  |
| 5. | Blank Contamination - If yes, list compounds and concentrations in each blank: |  |  |  |
| 6. | Matrix Spike/Matrix Spike Duplicate Recoveries Met Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. The Matrix Spike analysis met criteria for all parameters. The Matrix Spike Duplicate analysis met criteria for all parameters. |  |  |  |
| 7. | Sample Duplicate Analysis Met QC Criteria If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |
| 8. | Digestion Holding Time Met If not met, list number of days exceeded for each sample: |  |  |  |
| 9. | Analysis Holding Time Met If not met, list those compounds and their recoveries which fall outside the acceptable range. |  |  |  |

ADDITIONAL COMMENTS: Sample for Q2481-02,Q2481-03,Q2481-04,Q2481-06,Q2481-07, and Q2481-09 are analyzed straight as 10X dilution for TCLP Mercury due to very highly contaminated matrix and physical samples are dark and dark brown after digestion, not able to inject as straight to avoid damage to the instrument, can clog the tubes and carryover issue.

In analytical sequence LB136434, The Result was outside of acceptance limit for Silver of CCB08 but,  no any samples associated under this CCB.

FAX and Hard copy Data Not Match Due to at time of FAX analysis CCB fail for Silver parameter in sequence so Corrective action taken by Lab and Re-analyse in another sequence , so in Hard copy correct data reported.

Sample Q2481-01,Q2481-02,Q2481-03,and Q2481-09 are analyzed straight as 10X dilution, and sample Q2481-04,Q2481-05,Q2481-06,Q2481-07, and Q2481-10 are analyzed straight as 5X dilution for TCLP Metals Parameter due to ........

_______________________________________				__________________________

QA REVIEW								Date