CASE NARRATIVE

### Alliance Technical Group, LLC - Newark

### Project Name: NJ AIR PT

### Project # N/A

### Order ID # Q2727

Test Name: VOCMS Group3

A. Number of Samples and Date of Receipt:

1 Air sample was received on 07/30/2025.

B. Parameters

According to the Chain of Custody document, the following analyses were requested:

VOCMS Group3. This data package contains results for VOCMS Group3.

C. Analytical Techniques:

The analysis performed on instrument MSVOA_L were done using GC column RTX-1, which is 60 meters, 0.32 mm id, 1.0 um df, Restek Cat. #10157. The Trap was supplied by Entech, glass bead and Tenax , Entech 7100A Preconcentrator.The analysis of VOCMS Group3 was based on method TO-15.

D. QA/ QC Samples:

The Holding Times were met for all analysis.

The Surrogate recoveries were met for all analysis.

The Internal Standards Areas were met for all analysis.

The Retention Times were met for all analysis.

The RPD for {VL0731ABSD01} with File ID: VL042794.D met criteria except for Propene[28%] due to difference in results of BS and BSD.

The Blank Spike for {VL0731ABS01} with File ID: VL042790.D met requirements for all compounds except for Benzyl Chloride[134%] is failing high but no positive hit in associate sample therefore no corrective action taken.

The Blank Spike Duplicate for {VL0731ABSD01} with File ID: VL042794.D met requirements for all compounds except for Benzyl Chloride[136%] is failing high but no positive hit in associate sample therefore no corrective action taken.

The Blank analysis did not indicate the presence of lab contamination.

The %RSD is greater than 30% in the Initial Calibration method (VL072325AIR.M) for Ethanol, Naphthalene, and 1,2,4-Trichlorobenzene these compounds are passing on Linear Regression.

The Continuous Calibration File ID VL042788.D met the requirements except for Benzyl Chloride is failing high but no positive hit in associate sample therefore no corrective action taken.

The Tuning criteria met requirements.

Samples AE073-1100, AE073-1100DL were diluted due to high concentrations.

E. Additional Comments:

The Form 6 is not included in the data package because the Initial Calibration was performed using 7 points.

The not QT review data is reported in the Miscellaneous.

The Manual Integrations are performed for the followings.

| Manual Integration Report |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | VL072325 | Instrument | MSVOA_l |  |  |  |  |

| Sample ID | File ID | Parameter | Review By | Review On | Supervised By | Supervised On | Reason |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VSTDICCC010 | VL042747.D | m/p-Xylene | SAM | 7/24/2025 9:02:56 AM | MMDadoda | 7/24/2025 2:20:57 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | 1,1,2-Trichloroethane | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | 1,4-Dioxane | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | 4-Methyl-2-Pentanone | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | cis-1,3-Dichloropropene | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | Cyclohexane | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | Heptane | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | m/p-Xylene | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | t-1,3-Dichloropropene | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042748.D | trans-1,2-Dichloroethene | SAM | 7/24/2025 9:03:01 AM | MMDadoda | 7/24/2025 2:20:59 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | 1,4-Dioxane | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | 4-Methyl-2-Pentanone | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | Bromodichloromethane | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | cis-1,3-Dichloropropene | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | Ethanol | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | Heptane | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | m/p-Xylene | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | Methyl Methacrylate | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042749.D | t-1,3-Dichloropropene | SAM | 7/24/2025 9:03:21 AM | MMDadoda | 7/24/2025 2:21:02 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | 1,1,2-Trichloroethane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | 1,2-Dibromoethane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | 1,4-Dioxane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | 2-Hexanone | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | 4-Methyl-2-Pentanone | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Benzyl Chloride | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Bromodichloromethane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | cis-1,3-Dichloropropene | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Dibromochloromethane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Ethanol | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Heptane | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | m/p-Xylene | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Methyl Methacrylate | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | t-1,3-Dichloropropene | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Tetrahydrofuran | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042750.D | Trichloroethene | SAM | 7/24/2025 9:03:31 AM | MMDadoda | 7/24/2025 2:21:04 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042752.D | 1,1,1-Trichloroethane | SAM | 7/24/2025 9:03:05 AM | MMDadoda | 7/24/2025 2:21:09 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042752.D | 1,1,2,2-Tetrachloroethane | SAM | 7/24/2025 9:03:05 AM | MMDadoda | 7/24/2025 2:21:09 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042752.D | Carbon Tetrachloride | SAM | 7/24/2025 9:03:05 AM | MMDadoda | 7/24/2025 2:21:09 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042752.D | Tetrachloroethene | SAM | 7/24/2025 9:03:05 AM | MMDadoda | 7/24/2025 2:21:09 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042752.D | Trichloroethene | SAM | 7/24/2025 9:03:05 AM | MMDadoda | 7/24/2025 2:21:09 PM | Peak Integrated by Software incorrectly |
| VSTDICC015 | VL042753.D | m/p-Xylene | SAM | 7/24/2025 9:03:41 AM | MMDadoda | 7/24/2025 2:21:10 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042754.D | 1,1,2,2-Tetrachloroethane | SAM | 7/24/2025 9:03:10 AM | MMDadoda | 7/24/2025 2:21:12 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042754.D | 1,2-Dibromoethane | SAM | 7/24/2025 9:03:10 AM | MMDadoda | 7/24/2025 2:21:12 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042754.D | Carbon Tetrachloride | SAM | 7/24/2025 9:03:10 AM | MMDadoda | 7/24/2025 2:21:12 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042754.D | Tetrachloroethene | SAM | 7/24/2025 9:03:10 AM | MMDadoda | 7/24/2025 2:21:12 PM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042754.D | Trichloroethene | SAM | 7/24/2025 9:03:10 AM | MMDadoda | 7/24/2025 2:21:12 PM | Peak Integrated by Software incorrectly |
| VSTDICV010 | VL042755.D | 1,1,2-Trichloroethane | SAM | 7/24/2025 9:03:15 AM | MMDadoda | 7/24/2025 2:21:15 PM | Peak Integrated by Software incorrectly |
| VSTDICV010 | VL042755.D | m/p-Xylene | SAM | 7/24/2025 9:03:15 AM | MMDadoda | 7/24/2025 2:21:15 PM | Peak Integrated by Software incorrectly |

| Sequence |  | VL073125 |  | Instrument |  |  |  |  | MSVOA_l |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample ID | File ID |  | Parameter |  | Review By | Review On | Supervised By | Supervised On |  | Reason |  |
| VSTDCCC010 | VL042788.D |  | m/p-Xylene |  | SAM | 8/1/2025 10:51:05 AM | MMDadoda | 8/1/2025 4:16:30 PM |  | Peak Integrated by Software incorrectly |  |
| VL0731ABS01 | VL042790.D |  | m/p-Xylene |  | SAM | 8/1/2025 10:51:10 AM | MMDadoda | 8/1/2025 4:16:31 PM |  | Peak Integrated by Software incorrectly |  |
| VL0731ABSD01 | VL042794.D |  | m/p-Xylene |  | SAM | 8/1/2025 10:51:25 AM | MMDadoda | 8/1/2025 4:16:38 PM |  | Peak Integrated by Software incorrectly |  |

| Manual Integration Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sequence | VL081325 | Instrument | MSVOA_l |  |

| Sample ID | File ID | Parameter | Review By | Review On | Supervised By | Supervised On | Reason |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VSTDICCC010 | VL042842.D | m/p-Xylene | SAM | 8/14/2025 7:48:06 AM | MMDadoda | 8/18/2025 8:42:07 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | 1,1,2-Trichloroethane | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | 1,4-Dioxane | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | cis-1,3-Dichloropropene | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | Ethanol | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | m/p-Xylene | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC002 | VL042843.D | Methyl Methacrylate | SAM | 8/14/2025 7:48:12 AM | MMDadoda | 8/18/2025 8:42:11 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | 1,2-Dichloropropane | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | 1,4-Dioxane | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | 2,2,4-Trimethylpentane | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | Ethanol | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | Heptane | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | m/p-Xylene | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | Methyl Methacrylate | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | t-1,3-Dichloropropene | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC001 | VL042844.D | trans-1,2-Dichloroethene | SAM | 8/14/2025 7:48:18 AM | MMDadoda | 8/18/2025 8:42:13 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | 1,1,2-Trichloroethane | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | 1,4-Dioxane | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | 2,2,4-Trimethylpentane | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | 4-Methyl-2-Pentanone | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | cis-1,3-Dichloropropene | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | m/p-Xylene | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | Methyl Methacrylate | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | t-1,3-Dichloropropene | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.5 | VL042845.D | Toluene | SAM | 8/14/2025 7:49:25 AM | MMDadoda | 8/18/2025 8:42:16 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042846.D | 1,1,1-Trichloroethane | SAM | 8/14/2025 7:48:23 AM | MMDadoda | 8/18/2025 8:42:18 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042846.D | 1,2-Dibromoethane | SAM | 8/14/2025 7:48:23 AM | MMDadoda | 8/18/2025 8:42:18 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042846.D | Naphthalene | SAM | 8/14/2025 7:48:23 AM | MMDadoda | 8/18/2025 8:42:18 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042846.D | Tetrachloroethene | SAM | 8/14/2025 7:48:23 AM | MMDadoda | 8/18/2025 8:42:18 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.1 | VL042846.D | Trichloroethene | SAM | 8/14/2025 7:48:23 AM | MMDadoda | 8/18/2025 8:42:18 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042847.D | 1,1,1-Trichloroethane | SAM | 8/14/2025 7:49:29 AM | MMDadoda | 8/18/2025 8:42:20 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042847.D | 1,1,2,2-Tetrachloroethane | SAM | 8/14/2025 7:49:29 AM | MMDadoda | 8/18/2025 8:42:20 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042847.D | Carbon Tetrachloride | SAM | 8/14/2025 7:49:29 AM | MMDadoda | 8/18/2025 8:42:20 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042847.D | Tetrachloroethene | SAM | 8/14/2025 7:49:29 AM | MMDadoda | 8/18/2025 8:42:20 AM | Peak Integrated by Software incorrectly |
| VSTDICC0.03 | VL042847.D | Trichloroethene | SAM | 8/14/2025 7:49:29 AM | MMDadoda | 8/18/2025 8:42:20 AM | Peak Integrated by Software incorrectly |
| VSTDICC015 | VL042848.D | m/p-Xylene | SAM | 8/14/2025 7:49:35 AM | MMDadoda | 8/18/2025 8:42:35 AM | Peak Integrated by Software incorrectly |
| VSTDICV010 | VL042849.D | cis-1,3-Dichloropropene | SAM | 8/14/2025 7:48:28 AM | MMDadoda | 8/18/2025 8:42:37 AM | Peak Integrated by Software incorrectly |
| VSTDICV010 | VL042849.D | m/p-Xylene | SAM | 8/14/2025 7:48:28 AM | MMDadoda | 8/18/2025 8:42:37 AM | Peak Integrated by Software incorrectly |
| VL0813ABS01 | VL042851.D | 1,4-Dioxane | SAM | 8/14/2025 7:49:08 AM | MMDadoda | 8/18/2025 8:42:38 AM | Peak Integrated by Software incorrectly |
| VL0813ABS01 | VL042851.D | m/p-Xylene | SAM | 8/14/2025 7:49:08 AM | MMDadoda | 8/18/2025 8:42:38 AM | Peak Integrated by Software incorrectly |

F. Manual Integration Comments:

Please refer to the Manual integration Report included with the Run Logs for information on the manual integrations performed.

I certify that the data package is in compliance with the terms and conditions of the contract, both technically and for completeness, for other than the conditions detailed above. The laboratory manager or his designee, as verified by the following signature has authorized release of the data contained in this hard copy data package.

Signature__________________________