[image: image1.png]AII|anoe

ECHNICA





284 Sheffield Street, Mountainside, NJ 07092 Phone: 908 789 8900 Fax: 908 789 8922
CASE NARRATIVE
Tully Environmental, Inc
Project Name: Transfer Station-SPDES
Project # N/A
Order ID # Q3041
Test Name: Ammonia,BOD5,Oil and Grease,TSS
A. Number of Samples and Date of Receipt:

2 Water samples were received on 09/08/2025.

B. Parameters:


According to the Chain of Custody document, the following analyses were requested:

Ammonia,BOD5,Oil and Grease,TSS. This data package contains results for Ammonia,BOD5,Oil and Grease,TSS.

C. Analytical Techniques:

The analysis of Oil and Grease was based on method 1664A, The analysis of TSS was based on method SM2540 D, The analysis of Ammonia was based on method SM4500-NH3 and The analysis of BOD5 was based on method SM5210 B.
D. QA/ QC Samples:

The Holding Times were met for all samples except for 001 Willets Pt Blvd (Sep) of BOD5 and for 002 35th Ave(sep) of BOD5 as samples were receive out of holding time.
Sample 001 Willets Pt Blvd (Sep) was diluted due to high concentrations for Ammonia as N & Sample 002 35th Ave(sep) was diluted due to high concentrations for Ammonia as N.
The Blank Spike met requirements for all compounds.
The Duplicate analysis met criteria for all compounds.
The Matrix Spike analysis met criteria for all compounds.
The Matrix Spike Duplicate analysis met criteria for all compounds.
The Blank analysis did not indicate the presence of lab contamination.
The Calibration met the requirements.      

E. Additional Comments:
The temperature of the samples at the time of receipt was 20.1°C. Lab notified this issue to the client. See the communication in shipping Document section. 

As per method 1664A, MS/MSD is required to be performed with the sample analysis. However, Lab did not receive sufficient volume to perform the MS/MSD for Q3041 therefore Lab reported MS-MSD from Q3057.
[image: image1.png]
I certify that the data package is in compliance with the terms and conditions of the contract, both technically and for completeness, for other than the conditions detailed above. The laboratory manager or his designee, as verified by the following signature has authorized release of the data contained in this hard copy data package.

Signature__________________________


