[image: image1.png]AII|anoe

ECHNICA





284 Sheffield Street, Mountainside, NJ 07092 Phone: 908 789 8900 Fax: 908 789 8922
CASE NARRATIVE
Always Available Construction LLC.
Project Name: Yard Soil Cleanup
Project # N/A
Order ID # Q3157
Test Name: Mercury,Metals ICP-TAL
A. Number of Samples and Date of Receipt:

2 Solid samples were received on 09/19/2025.

1 Water sample was received on 09/19/2025.
B. Parameters:


According to the Chain of Custody document, the following analyses were requested:

Mercury,Metals ICP-TAL. This data package contains results for Mercury,Metals ICP-TAL.

C. Analytical Techniques:

The analysis of Metals ICP-TAL was based on method 6010D, digestion based on method 3050 (soils). The analysis and digestion of Mercury was based on method 7471B.
D. QA/ QC Samples:

The Holding Times were met for all analysis.
The Blank Spike met requirements for all compounds.
The Duplicate (20-DEAD-1DUP) analysis met criteria for all compounds except for Calcium and Lead due to sample matrix interference.
The Matrix Spike (20-DEAD-1MS) analysis met criteria for all compounds except for Antimony and Lead due to Chemical Interference during Digestion Process.
The Matrix Spike Duplicate (20-DEAD-1MSD) analysis met criteria for all compounds except for Antimony, Copper, Lead and Vanadium due to Chemical Interference during Digestion Process. The Matrix Spike Duplicate (VNJ-255MSD) analysis met criteria for all compounds except for Mercury due to sample matrix interference.
The Blank analysis did not indicate the presence of lab contamination.
The Calibration met the requirements.      

The Serial Dilution (20-DEAD-1L) met criteria for all compounds except for Aluminum, Chromium, Copper, Iron, Magnesium, Manganese, Potassium and Zinc due to unknow sample matrix interference. 
E. Additional Comments:
The temperature of the samples at the time of receipt was 26.1°C
[image: image1.png]
I certify that the data package is in compliance with the terms and conditions of the contract, both technically and for completeness, for other than the conditions detailed above. The laboratory manager or his designee, as verified by the following signature has authorized release of the data contained in this hard copy data package.

Signature__________________________


