[image: image1.png]AII|anoe

ECHNICA





284 Sheffield Street, Mountainside, NJ 07092 Phone: 908 789 8900 Fax: 908 789 8922
CASE NARRATIVE
Lockwood, Kessler & Bartlett, Inc.
Project Name: Syosset Landfill 2025
Project # N/A
Order ID # Q3202
Test Name: Alkalinity,Ammonia,Anions Group1,BOD5,pH,TDS,TKN,Total  Nitrogen,TSS,Turbidity
A. Number of Samples and Date of Receipt:

9 Water samples were received on 09/24/2025.

B. Parameters:


According to the Chain of Custody document, the following analyses were requested:

Alkalinity,Ammonia,Anions Group1,BOD5,pH,TDS,TKN,Total  Nitrogen, TSS, Turbidity. This data package contains results for Alkalinity,Ammonia,Anions Group1, BOD5,pH,TDS,TKN,Total  Nitrogen,TSS,Turbidity.

C. Analytical Techniques:

The analysis of Anions Group1 was based on method 300.0, The analysis of Total  Nitrogen was based on method Cal, The analysis of pH was based on method SM 4500-H B, The analysis of Turbidity was based on method SM2130 B, The analysis of Alkalinity was based on method SM2320 B, The analysis of TDS was based on method SM2540 C, The analysis of TSS was based on method SM2540 D, The analysis of TKN was based on method SM4500 N Org B or C, The analysis of Ammonia was based on method SM4500-NH3 and The analysis of BOD5 was based on method SM5210 B.
D. QA/ QC Samples:

The Holding Times were met for all samples except for SY-2D of pH, for SY-2R of pH, for SY-3 of pH, for SY-3D of pH, for SY-3DD of pH, for SY-5 of pH, for SY-6 of pH as samples were receive out of holding time. The Holding Times were met for all samples except for SY-6 of Nitrate and Nitrite as sample was analyzed out of holding time. The Holding Times were met for all samples except for SY-6 of TDS, for SY-2R of TDS as samples were analyzed out of holding time.
Sample SY-2R was diluted due to high concentrations for Chloride & Sample SY-2D was diluted due to high concentrations for Chloride & Sample SY-5 was diluted due to high concentrations for Chloride & Sample SY-3D was diluted due to high concentrations for Ammonia as N,TKN,Chloride,Sulfate & Sample SY-3DDL was diluted due to high concentrations for Chloride & Sample SY-3 was diluted due to high concentrations for Ammonia as N,Chloride.
The Blank Spike met requirements for all compounds.
The Duplicate analysis met criteria for all compounds.
The Matrix Spike (SY-3DMS) analysis met criteria for all compounds except for Chloride due to sample matrix interference.
The Matrix Spike Duplicate (SY-3DMSD) analysis met criteria for all compounds except for Chloride due to sample matrix interference.
The Blank analysis did not indicate the presence of lab contamination.
The Calibration met the requirements.      

E. Additional Comments:
Due to very turbid matrix Q3202-11 was performed with 2X straight dilution and Q3202-17 was performed with 100X straight dilution for Turbidity parameter.
[image: image1.png]
I certify that the data package is in compliance with the terms and conditions of the contract, both technically and for completeness, for other than the conditions detailed above. The laboratory manager or his designee, as verified by the following signature has authorized release of the data contained in this hard copy data package.

Signature__________________________


