ALLIANCE 284 Sheffield Street, Mountainside New Jersey 07092

NEW JERSEY LAB ID#: 20012: NEW YORK LAB ID#: 11376

GC/MS VOA CONFORMANCE/NON-CONFORMANCE SUMMARY
ALLIANCE 284 Sheffield Street, Mountainside New Jersey 07092

NEW JERSEY LAB ID#: 20012: NEW YORK LAB ID#: 11376

GC/MS VOA CONFORMANCE/NON-CONFORMANCE SUMMARY (CONTINUED)
	
	
	NA
	NO
	YES



	ORDER ID: Q3248

	MATRIX: Water

	METHOD: 8260D

	


	
	
	NA
	NO
	YES

	1.
	Chromatograms Labeled/Compounds Identified.  (Field samples and Method Blanks)


	
	
	(

	2.
	GC/MS  Tuning Specifications

BFB Meet Criteria (NOTE THAT THERE ARE DIFFERENT CRITERIA FOR NY 

ASP CLP, CLP AND NJ)


	
	
	(

	3.
	GC/MS Tuning Frequency - Performed every 24 hours for 600 series and 12 hours for 8000 Series.


	
	
	(

	4.
	GC/MS Calibration - Initial Calibration performed before sample analysis and continuing calibration performed within 24 hours of sample analysis for 600 series and 12 hours for 8000 series.


	
	
	(

	5.
	GC/MS Calibration Requirements.
	
	
	(

	
	The Initial Calibration met the requirements.

The Continuous Calibration met the requirements.
	
	
	

	6.
	Blank Contamination - If yes, list compounds and concentrations in each blank:


	
	(
	

	7.
	Surrogate Recoveries Meet Criteria


If not met, list those compounds and their recoveries which fall outside the acceptable ranges.


	
	
	(

	8.
	Matrix Spike/Matrix Spike Duplicate Recoveries Meet Criteria

If not met, list those compounds and their recoveries which fall outside the acceptable range.

The MS {Q3248-09MS} with File ID: VX048005.D recoveries met the requirements for all compounds except for 1,2,4-Trichlorobenzene[68%], 1,2-Dichlorobenzene[75%], 1,3-Dichlorobenzene[73%] and 1,4-Dichlorobenzene[73%] due to matrix interference.

The RPD for {Q3248-10MSD} with File ID: VX048006.D met criteria except for 1,1,2,2-Tetrachloroethane[29%], 1,2,3-Trichlorobenzene[32%], 1,2,4-Trichlorobenzene[31%], 1,2-Dibromo-3-Chloropropane[31%], 1,2-Dibromoethane[23%], 1,2-Dichlorobenzene[31%], 1,3-Dichlorobenzene[32%], 1,4-Dichlorobenzene[30%], Bromodichloromethane[21%], Bromoform[28%], Chlorobenzene[26%], cis-1,3-Dichloropropene[21%], Ethyl Benzene[26%], Isopropylbenzene[30%], m/p-Xylenes[30%], o-Xylene[27%], Styrene[27%], t-1,3-Dichloropropene[22%], Tetrachloroethene[24%] and Toluene[21%] due to difference in results of MS and MSD.

	
	(
	

	9.
	Internal Standard Area/Retention Time Shift Meet Criteria

Comments: 


	
	
	(

	10.
	Analysis Holding Time Met 






If not met, list number of days exceeded for each sample:


	
	
	(


ADDITIONAL COMMENTS: 
The not QT review data is reported in the Miscellaneous.
The laboratory certifies that the all-electronic diskette deliverable exactly match the data

Summary forms (i.e. Form Is).”
The sample # BP-VPB-184-GW-685-687MS is failing for 1,2,4-Trichlorobenzene, 1,2-Dichlorobenzene, 1,3-Dichlorobenzene and 1,4-Dichlorobenzene therefore the original sample(# BP-VPB-184-GW-685-687) is reported with M flag for this compounds.
_______________________________________



__________________________

QA REVIEW







Date

