ALLIANCE 284 Sheffield Street, Mountainside New Jersey 07092

NEW JERSEY LAB ID#: 20012: NEW YORK LAB ID#: 11376

METALS CONFORMANCE/NON-CONFORMANCE SUMMARY
ALLIANCE 284 Sheffield Street, Mountainside New Jersey 07092

NEW JERSEY LAB ID#: 20012: NEW YORK LAB ID#: 11376

METALS CONFORMANCE/NON-CONFORMANCE SUMMARY (CONTINUED)
	
	
	NA
	NO
	YES



	ORDER ID: Q3739

	MATRIX: Solid

	METHOD: 6010D,7471B

	


	
	
	NA
	NO
	YES

	   1.
	Calibration Summary met criteria.


	
	
	(

	   2.
	ICP Interference Check Sample Results Summary Submitted.


	
	
	(

	   3.
	Serial Dilution Summary (if applicable) Submitted.


	
	
	(

	   4.
	Laboratory Control Sample Summary (if applicable) Submitted.


	
	
	(

	   5.
	Blank Contamination - If yes, list compounds and concentrations in each blank:



	
	(
	

	   6.
	Matrix Spike/Matrix Spike Duplicate Recoveries Met Criteria

If not met, list those compounds and their recoveries which fall outside the acceptable range.


	
	
	(

	   7.
	Sample Duplicate Analysis Met QC Criteria

If not met, list those compounds and their recoveries which fall outside the acceptable range.


	
	
	(

	   8.
	Digestion Holding Time Met 






If not met, list number of days exceeded for each sample:


	
	
	(

	   9.
	Analysis Holding Time Met 






If not met, list those compounds and their recoveries which fall outside the acceptable range.


	
	
	(


ADDITIONAL COMMENTS: The Post Digest Spike (WC1A) analysis met criteria for all compounds except for Antimony, Potassium, Vanadium due to unknown chemical interference of matrix with the addition of spike amount after digestion and before analysis; matrix has suppression effect during addition of spike.

All samples and QC samples have very greenish type of digested sample, looks vey highly unknown contaminations and may be some type of organic interferences which can cause instrument failure for QC, due to suppression effect and could cause sample introduction issue and carryover problem. So straight 5X dilution analyzed and reported for samples.
_______________________________________



__________________________

QA REVIEW







Date


